Eurobot Manipulation Package Documentation

1. Project Overview

1.1 High-Level Goal

The eurobot_manipulator package provides gripper control functionality for the Eurobot autonomous robot. It enables the robot to:

- Grasp colored crates detected by the perception system

- Hold objects securely during navigation

- Release crates at designated pantry locations

- Detect contact using finger-mounted sensors to prevent over-gripping

- Coordinate with the task manager for autonomous pick-and-place operations

- Respond to both autonomous commands and manual control inputs

The gripper uses a parallel-jaw design with two finger joints, controlled via position commands and equipped with contact sensors for tactile feedback.

1.2 System Architecture

The manipulation package consists of a single intelligent node:

gripper_controller - State machine-based gripper controller with contact sensing

The controller implements a robust state machine that manages gripper behavior:

- Initializing - Startup sequence, opens gripper to home position

- Idle - Ready to receive commands

- Closing - Actively closing until contact or limit reached

- Gripping - Holding an object with sustained grip force

- Opening - Actively opening to release object

1.3 Hardware and Simulation Environment

- Platform: ROS 2 Humble

- Simulation: Gazebo with ros_gz_bridge

- Actuators: 2 position-controlled finger joints (left_finger_joint, right_finger_joint)

- Sensors: Contact sensors on both gripper fingers

- Control Rate: 20 Hz command loop

- Controller: ros2_control with JointGroupPositionController

1.4 Design Philosophy

The gripper controller is designed with robustness and safety in mind:

- Contact-based feedback: Stops closing when object is detected to prevent damage

- State machine logic: Clear, deterministic behavior transitions

- Dual command interface: Supports both autonomous (task_manager) and manual control

- Fail-safe behavior: Detects empty grasps and returns to idle state

- Real-time monitoring: Publishes gripper state for system awareness

2. Package Summary

Package Name

eurobot_manipulator

Goal

Provide reliable, contact-sensing gripper control for autonomous crate manipulation in the Eurobot competition.

Dependencies

ROS 2 Packages:

- rclpy - ROS 2 Python client library

- std_msgs - String messages for commands and state

- sensor_msgs - JointState messages for position feedback

- geometry_msgs - (available for future use)

- control_msgs - Control system interfaces

- trajectory_msgs - Trajectory control (available for future use)

- ros_gz_interfaces - Gazebo contact sensor messages

External Systems:

- ros2_control - Controller manager and interfaces

- controller_manager - Runtime controller management

- position_controllers - JointGroupPositionController

Main Nodes

| Node Name | Purpose | Type |
| --- | --- | --- |
| gripper_controller | State machine-based gripper control with contact sensing | Full Implementation |

Configuration Files

gripper_controller.yaml - Contains configuration for:

- Controller manager parameters (update rate: 100 Hz)

- Joint state broadcaster configuration

- Gripper controller parameters (joints, interfaces, rates)

Interaction with Other Packages

Integration with eurobot_navigation:

- Subscribes to /manipulation/command (CrateManipulation) - planned integration

- Subscribes to /gripper/command (String) - direct manual control

- Publishes /gripper/state (String) - gripper status for monitoring

Hardware Interface:

- Publishes to /gripper_controller/commands (Float64MultiArray) - position commands

- Subscribes to /joint_states (JointState) - current finger positions

- Subscribes to /gripper/left/contact and /gripper/right/contact (Contacts) - tactile feedback

3. Node-Level Documentation

3.1 Node: gripper_controller

Node Name

gripper_controller_node

Goal / Functionality

Implements a state machine-based gripper controller that manages parallel-jaw gripper operation with contact sensing. The node:

- Controls finger positions via incremental position commands

- Monitors contact sensors on both fingers to detect object grasping

- Implements state machine for robust grip/release behavior

- Prevents over-gripping by stopping closure when contact is detected

- Publishes state for system monitoring and coordination

- Supports dual command interface for autonomous and manual control

Subscribed Topics

| Topic | Type | Purpose |
| --- | --- | --- |
| /gripper/command | std_msgs/String | Manual control commands ("open", "close") |
| /manipulation/command | eurobot_interfaces/CrateManipulation | Autonomous commands from task_manager (planned) |
| /joint_states | sensor_msgs/JointState | Current finger joint positions and velocities |
| /gripper/left/contact | ros_gz_interfaces/Contacts | Left finger contact sensor data |
| /gripper/right/contact | ros_gz_interfaces/Contacts | Right finger contact sensor data |

Published Topics

| Topic | Type | Purpose |
| --- | --- | --- |
| /gripper_controller/commands | std_msgs/Float64MultiArray | Position commands [left_pos, right_pos] |
| /gripper/state | std_msgs/String | Current gripper state for monitoring |

Parameters

Currently no ROS parameters - all values are hardcoded constants:

| Constant | Value | Description |
| --- | --- | --- |
| min_pos | -0.03 | Minimum finger position (fully closed) |
| max_pos | 0.03 | Maximum finger position (fully open) |
| step | 0.005 | Position increment per control cycle |
| Control rate | 20 Hz | Main control loop frequency (0.05s timer) |
| Init rate | 1 Hz | Initialization timer frequency |

State Machine

The gripper operates in five distinct states:

- INITIALIZING

  - Initial state on startup

  - Waits 1 second, then sends OPEN command [0.03, 0.03]

  - Transitions to IDLE

- IDLE

  - Ready to receive commands

  - No motion, maintains current position

  - Accepts "open" or "close" commands

- CLOSING

  - Incrementally decreases finger positions (step = -0.005 per cycle)

  - Monitors contact sensors continuously

  - Stops individual finger when contact detected

  - Transitions to GRIPPING when both fingers detect contact

  - Transitions to IDLE if fully closed without contact (empty grasp warning)

- GRIPPING

  - Maintains current position to hold object

  - No further motion

  - Accepts "open" command to release

- OPENING

  - Incrementally increases finger positions (step = +0.005 per cycle)

  - Continues until max_pos (0.03) reached

  - Transitions to IDLE when fully open

  - Can be commanded from IDLE, GRIPPING, or CLOSING states

Internal State Variables

# Sensor feedback

left_contact: bool          # True if left finger detects contact

right_contact: bool         # True if right finger detects contact

# Joint positions (from /joint_states)

left_pos: float            # Current left finger position

right_pos: float           # Current right finger position

joint_initialized: bool    # True after first joint_states received

# State machine

state: str                 # "initializing", "idle", "closing", "gripping", "opening"

# Initialization

init_sent: bool           # True after initialization command sent

Algorithm Flow

Initialization Sequence:

def initialize_gripper():

IF not init_sent:

# Send open command

publish [0.03, 0.03] to /gripper_controller/commands

LOG: "Gripper initialized to OPEN"

state = "idle"

init_sent = True

cancel init_timer

Command Callback (command_callback):

def command_callback(msg):

command = msg.data.lower()

IF command == "close":

close_gripper()

ELSE IF command == "open":

open_gripper()

ELSE:

WARN: "Unknown gripper command: {command}"

Contact Sensor Callbacks:

def left_contact_cb(msg):

left_contact = (len(msg.contacts) > 0)

IF left_contact AND state == "closing":

LOG: "Left finger contact!" (throttled to 1Hz)

def right_contact_cb(msg):

right_contact = (len(msg.contacts) > 0)

IF right_contact AND state == "closing":

LOG: "Right finger contact!" (throttled to 1Hz)

Joint State Callback:

def joint_state_cb(msg):

TRY:

idx_left = msg.name.index('left_finger_joint')

idx_right = msg.name.index('right_finger_joint')

left_pos = msg.position[idx_left]

right_pos = msg.position[idx_right]

IF not joint_initialized:

joint_initialized = True

LOG: "Joint states: left={left_pos}, right={right_pos}"

EXCEPT (ValueError, IndexError):

pass  # Joint names not found

Main Control Loop (20 Hz):

def control_loop():

# 1. Publish current state

publish state to /gripper/state

# 2. State-specific behavior

IF state == "initializing":

return  # Wait for initialization

ELSE IF state == "closing":

# Check if grasp achieved

IF left_contact AND right_contact:

state = "gripping"

LOG: "Crate grasped!"

return

# Incrementally close each finger until contact

IF not left_contact AND left_pos > min_pos:

left_pos -= step  # Decrease by 0.005

IF not right_contact AND right_pos > min_pos:

right_pos -= step

# Check if fully closed without contact (empty grasp)

IF left_pos <= min_pos AND right_pos <= min_pos:

IF not (left_contact AND right_contact):

WARN: "Gripper fully closed but no object!"

state = "idle"

ELSE IF state == "opening":

# Incrementally open

IF left_pos < max_pos:

left_pos += step

IF right_pos < max_pos:

right_pos += step

# Check if fully open

IF left_pos >= max_pos AND right_pos >= max_pos:

LOG: "Gripper fully opened"

state = "idle"

ELSE IF state == "gripping":

pass  # Hold current position

ELSE IF state == "idle":

pass  # No motion

# 3. Clamp positions to valid range

left_pos = clamp(left_pos, min_pos, max_pos)

right_pos = clamp(right_pos, min_pos, max_pos)

# 4. Publish position commands

publish [left_pos, right_pos] to /gripper_controller/commands

Close Gripper:

def close_gripper():

IF state in ["idle", "opening"]:

LOG: "Closing gripper..."

state = "closing"

left_contact = False

right_contact = False

ELSE:

WARN: "Cannot close in state: {state}"

Open Gripper:

def open_gripper():

IF state in ["idle", "gripping", "closing"]:

LOG: "Opening gripper..."

state = "opening"

left_contact = False

right_contact = False

ELSE:

WARN: "Cannot open in state: {state}"

Contact Sensing Logic

The gripper uses contact sensors to implement adaptive grasping:

- During CLOSING:

  - Each finger closes independently until contact is detected

  - Contact detection: len(msg.contacts) > 0

  - When a finger touches the crate, it stops moving

  - The other finger continues until it also makes contact

  - This allows grasping objects that may not be perfectly centered

- Grasp Confirmation:

  - State transitions to GRIPPING only when both fingers detect contact

  - This ensures a secure two-point grasp

- Empty Grasp Detection:

  - If both fingers reach min_pos (-0.03) without contact

  - Log warning: "Gripper fully closed but no object!"

  - Transition to IDLE (ready for retry)

- Contact Reset:

  - Contact flags reset to False when entering CLOSING or OPENING states

  - Prevents false positives from previous grasps

4. Topic Communication Summary

4.1 Node Communication Table

| Node | Publishes | Subscribes | Message Type | Description |
| --- | --- | --- | --- | --- |
| gripper_controller | /gripper_controller/commands | /gripper/command | Float64MultiArray | Position commands [left, right] |
|  | /gripper/state | /manipulation/command | String | Current gripper state |
|  |  | /joint_states | CrateManipulation (planned) | Commands from task_manager |
|  |  | /gripper/left/contact | JointState | Joint position feedback |
|  |  | /gripper/right/contact | Contacts | Contact sensor data |

4.2 Message Format Details

Float64MultiArray (Position Commands)

# Published to /gripper_controller/commands

data: [left_finger_pos, right_finger_pos]  # Both in range [-0.03, 0.03]

String (Command Input)

# Subscribed from /gripper/command

data: "open" or "close"  # Case-insensitive

String (State Output)

# Published to /gripper/state

data: "initializing" | "idle" | "closing" | "gripping" | "opening"

Contacts (Contact Sensors)

# Subscribed from /gripper/{left|right}/contact

contacts: [

# List of Contact messages, empty if no contact

{

collision1_name: "gripper::left_finger::collision",

collision2_name: "crate::collision",

position: Point3D,

normal: Vector3D,

depth: float

}

]

# Contact detected if: len(msg.contacts) > 0

JointState (Position Feedback)

# Subscribed from /joint_states

name: ["left_finger_joint", "right_finger_joint", ...]

position: [0.025, 0.020, ...]  # Corresponding positions

velocity: [0.0, 0.0, ...]

effort: [0.0, 0.0, ...]

4.3 Integration Topics (Planned)

CrateManipulation (from task_manager)

# Will subscribe to /manipulation/command

int32 crate_id    # ID of crate being manipulated

int32 command     # 0=PICK, 1=PLACE

Planned Integration Logic:

def manipulation_command_cb(msg):

IF msg.command == 0:  # PICK

close_gripper()

LOG: "Picking crate {msg.crate_id}"

ELSE IF msg.command == 1:  # PLACE

open_gripper()

LOG: "Placing crate {msg.crate_id}"

5. Configuration Details

5.1 gripper_controller.yaml

File Location: eurobot_manipulator/config/gripper_controller.yaml

Controller Manager Configuration

controller_manager:

ros__parameters:

update_rate: 100  # Hz - High rate for smooth control

use_sim_time: true  # Using Gazebo simulation

# Registered controllers

joint_state_broadcaster:

type: joint_state_broadcaster/JointStateBroadcaster

gripper_controller:

type: position_controllers/JointGroupPositionController

Parameters Explained:

- update_rate: 100 - Controller manager runs at 100Hz for responsive control

- use_sim_time: true - Synchronize with Gazebo simulation time

- Controller types registered for spawning at runtime

Joint State Broadcaster

joint_state_broadcaster:

ros__parameters:

use_sim_time: true

Purpose: Publishes /joint_states topic containing current positions, velocities, and efforts of all joints.

Gripper Controller

gripper_controller:

ros__parameters:

use_sim_time: true

joints:

- left_finger_joint

- right_finger_joint

command_interfaces:

- position

state_interfaces:

- position

- velocity

allow_partial_joints_goal: false

state_publish_rate: 50.0

action_monitor_rate: 20.0

Parameters Explained:

| Parameter | Value | Purpose |
| --- | --- | --- |
| joints | [left_finger_joint, right_finger_joint] | Controlled joint names (must match URDF) |
| `command_ |  |  |