Eurobot Robot Model Documentation

Physical Design & Gazebo Simulation Configuration

1. Robot Overview

1.1 High-Level Description

The simple_robot is a differential-drive mobile manipulator designed for the Eurobot competition. It features:

- Mobile base: Differential drive with two powered wheels and one passive caster

- Manipulator: Parallel-jaw gripper for picking and placing crates

- Sensors: 2D LiDAR for mapping/navigation and RGB camera for object detection

- Simulation: Full Gazebo integration with physics, sensors, and control

1.2 Robot Dimensions

| Component | Dimension | Value |
| --- | --- | --- |
| Base Length | X-axis | 280 mm (0.28 m) |
| Base Width | Y-axis | 250 mm (0.25 m) |
| Base Height | Z-axis | 70 mm (0.07 m) |
| Total Mass | - | ~3.86 kg |
| Wheelbase | Center-to-wheel | 140 mm (0.14 m) per side |
| Ground Clearance | - | 15 mm (wheel bottom) |

1.3 Design Philosophy

The robot is designed for:

- Stability: Low center of gravity with wide wheelbase

- Maneuverability: Differential drive for precise turning

- Manipulation: Front-mounted gripper optimized for 150mm x 100mm x 30mm crates

- Perception: 360° LiDAR coverage and forward-facing camera

- Modularity: Clean separation between base, sensors, and manipulator

2. Mechanical Design

2.1 Overall Layout

2.2 Component Positioning

Base Link (Center of robot coordinate system):

- Position: (0, 0, 0.025) - Base center at wheel axle height (25mm)

- This places wheel centers at ground level (z=0)

Gripper Base:

- Position: (0.17, 0, -0.02) relative to base_link

- 17cm forward from robot center (front-mounted)

- 2cm below base center for optimal crate pickup height

Wheels:

- Left: (-0.06, 0.14, 0) - 6cm behind center, 14cm left

- Right: (-0.06, -0.14, 0) - 6cm behind center, 14cm right

- Positioned 80mm from rear edge for weight distribution

Caster:

- Position: (0.11, 0, -0.02) - 11cm forward, 2cm below base

- Provides front support and smooth rotation

Sensors:

- LiDAR: (0, 0, 0.05) - Centered on top of base, 5cm high

- Camera: (0.10, 0, 0.20) - 10cm forward, 20cm high, tilted down 0.7 rad (40°)

3. Link Descriptions

3.1 Base Link

Name: base_link (canonical link)

Purpose: Main robot body, serves as primary reference frame

Geometry:

- Shape: Box

- Dimensions: 280mm × 250mm × 70mm (L × W × H)

- Position: (0, 0, 0.025) relative to model origin

Inertial Properties:

Mass: 3.0 kg

Inertia Tensor:

Ixx = 0.0186 kg⋅m²

Iyy = 0.0228 kg⋅m²

Izz = 0.0391 kg⋅m²

Material: Blue color (RGB: 0.2, 0.2, 0.8)

Collision: Full box collision for obstacle detection

3.2 Base Footprint

Name: base_footprint

Purpose: Ground-level reference frame for navigation

Position: (0, 0, -0.025) relative to base_link (at ground level z=0)

Type: Virtual link (no geometry or mass)

Connection: Fixed joint to base_link

3.3 Wheel Links

Left Wheel

Name: left_wheel

Geometry:

- Shape: Cylinder

- Radius: 40mm (80mm diameter)

- Width: 30mm

Position: (-0.06, 0.14, 0) relative to base_link

- Orientation: Rotated -90° (π/2 rad) around X-axis to align with Y-axis motion

Inertial Properties:

Mass: 0.3 kg

Inertia: Ixx = Iyy = 0.00024 kg⋅m², Izz = 0.00048 kg⋅m²

Surface Properties:

- Friction: μ = μ₂ = 1.0 (high traction)

- Contact stiffness: kp = 1×10⁶ N/m

- Contact damping: kd = 100 N⋅s/m

- No slip allowed

Right Wheel

Name: right_wheel

Specifications: Identical to left wheel

Position: (-0.06, -0.14, 0) (mirrored on Y-axis)

3.4 Caster Wheel

Name: caster

Purpose: Passive front support, allows smooth rotation

Geometry:

- Shape: Sphere

- Radius: 20mm (40mm diameter)

Position: (0.11, 0, -0.02) relative to base_link

- 110mm forward from center (30mm from front edge)

- 20mm below base center (matches wheel ground clearance)

Inertial Properties:

Mass: 0.05 kg

Inertia: All components = 0.00008 kg⋅m²

Surface Properties:

- Friction: μ = μ₂ = 0.01 (very low for free rolling)

- Slip: slip₁ = slip₂ = 1.0 (allows omnidirectional motion)

3.5 Gripper Links

Gripper Base

Name: gripper_base

Purpose: Mounting platform for finger actuators

Geometry:

- Shape: Box

- Dimensions: 80mm × 200mm × 30mm

Position: (0.17, 0, -0.02) relative to base_link

- Mounted at front of robot

- Height optimized for 30mm tall crate pickup

Inertial Properties:

Mass: 0.3 kg

Inertia: All components ≈ 0.0002 kg⋅m²

Material: Gray/silver (RGB: 0.7-0.9)

Left Finger

Name: left_finger

Geometry:

- Shape: Box

- Dimensions: 160mm × 10mm × 20mm (L × W × H)

Position: (0.04, 0.055, 0.0) relative to gripper_base

- 4cm forward, 5.5cm left of gripper center

- Positioned to grip 150mm (long side) of crate

Inertial Properties:

Mass: 0.08 kg

Inertia: Ixx = Izz = 0.00008, Iyy = 0.00002 kg⋅m²

Material: Red (RGB: 0.9, 0.3, 0.3)

Surface Properties:

- High friction: μ = μ₂ = 8.0 (prevents slippage during gripping)

- Contact stiffness: kp = 1×10⁶ N/m

- Max contact velocity: 0.01 m/s

- Min penetration depth: 0.001 m

Sensor: Contact sensor (left_finger_contact)

- Type: Contact detection

- Update rate: 200 Hz

- Topic: /gripper/left/contact

Right Finger

Name: right_finger

Specifications: Identical to left finger

Position: (0.04, -0.055, 0.0) (mirrored on Y-axis)

Sensor: Contact sensor (right_finger_contact)

- Topic: /gripper/right/contact

3.6 Sensor Links

LiDAR Link

Name: lidar_link

Purpose: 2D laser scanner mounting point

Geometry:

- Shape: Cylinder

- Radius: 25mm

- Height: 40mm

Position: (0, 0, 0.05) relative to base_link

- Centered on top of base

- 50mm above base center for clear 360° view

Inertial Properties:

Mass: 0.1 kg

Inertia: All components = 0.00002 kg⋅m²

Sensor Configuration: See Section 5.1

Camera Link

Name: camera_link

Purpose: RGB camera mounting point

Geometry:

- Shape: Box (small cube)

- Dimensions: 30mm × 30mm × 30mm

Position: (0.10, 0, 0.20) relative to base_link

- 10cm forward

- 20cm high

- Pitch angle: 0.7 rad (≈40° downward tilt)

Inertial Properties:

Mass: 0.05 kg

Inertia: All components = 0.00001 kg⋅m²

Sensor Configuration: See Section 5.2

4. Joint Specifications

4.1 Wheel Joints

Left Wheel Joint

Name: left_wheel_joint

Type: Revolute (continuous rotation)

Parent: base_link
Child: left_wheel

Axis: (0, 0, 1) - Rotation around Z-axis (after link orientation)

Limits:

- Lower: -1×10¹⁶ rad (effectively unlimited)

- Upper: +1×10¹⁶ rad

- Effort: 10 N⋅m (URDF), unlimited (SDF)

- Velocity: 10 rad/s (URDF), unlimited (SDF)

Control: Velocity-controlled via diff_drive plugin

Right Wheel Joint

Name: right_wheel_joint

Specifications: Identical to left wheel joint (mirrored)

4.2 Gripper Joints

Left Finger Joint

Name: left_finger_joint

Type: Prismatic (linear motion)

Parent: gripper_base
Child: left_finger

Axis: (0, 1, 0) - Motion along Y-axis (lateral)

Limits:

Lower:    -0.03 m  (closed position, 3cm inward)

Upper:    +0.03 m  (open position, 3cm outward)

Effort:   40 N     (maximum force)

Velocity: 0.05 m/s (5 cm/s max speed)

Dynamics:

- Damping: 3.0 N⋅s/m (smooth motion)

- Friction: 0.5 N (realistic resistance)

Control Interface: Position-controlled via ros2_control

- Command: Position setpoint (-0.03 to +0.03 m)

- State: Position, velocity, effort feedback

Right Finger Joint

Name: right_finger_joint

Type: Prismatic

Axis: (0, -1, 0) - Opposite direction (symmetric closing)

Limits: Same as left finger (-0.03 to +0.03 m)

Note: Negative Y-axis ensures both fingers move toward center when closing

4.3 Fixed Joints

| Joint Name | Parent | Child | Purpose |
| --- | --- | --- | --- |
| base_to_footprint | base_link | base_footprint | Navigation reference |
| caster_joint | base_link | caster | Front support |
| gripper_base_joint | base_link | gripper_base | Gripper mounting |
| lidar_joint | base_link | lidar_link | Sensor mounting |
| camera_joint | base_link | camera_link | Sensor mounting |

5. Sensor Configuration

5.1 LiDAR Sensor

Sensor Name: lidar_sensor

Type: gpu_lidar (GPU-accelerated ray tracing)

Topic: /scan

Frame: lidar_link

Update Rate: 10 Hz

Configuration:

Horizontal Scan:

Samples: 360 (1° resolution)

Range: -π to +π rad (full 360°)

Vertical Scan:

Samples: 1 (2D planar scan)

Range:

Minimum: 0.08 m (8 cm)

Maximum: 10.0 m

Resolution: 0.01 m (1 cm)

Noise:

Type: Gaussian

Mean: 0.0 m

Std Dev: 0.01 m (1 cm)

Always On: Yes
Visualization: Enabled

Published Message: sensor_msgs/LaserScan

ranges: float32[360]  # Distance measurements

angle_min: -3.14159

angle_max: 3.14159

angle_increment: 0.01745 (1°)

range_min: 0.08

range_max: 10.0

Use Cases:

- SLAM mapping (slam_toolbox)

- Localization (AMCL)

- Obstacle detection (Nav2 costmaps)

5.2 Camera Sensor

Sensor Name: camera_sensor

Type: camera (RGB image)

Topic: /camera

Frame: camera_link

Update Rate: 30 Hz

Configuration:

Field of View:

Horizontal FOV: 1.047 rad (60°)

Image:

Width: 320 pixels

Height: 240 pixels

Format: R8G8B8 (RGB, 8-bit per channel)

Clipping Planes:

Near: 0.1 m

Far: 100 m

Noise:

Type: Gaussian

Mean: 0.0

Std Dev: 0.007 (per pixel intensity)

Always On: Yes
Visualization: Enabled

Published Messages:

- /camera/image_raw (sensor_msgs/Image)

- /camera/camera_info (sensor_msgs/CameraInfo)

Mounting:

- Tilted 0.7 rad (40°) downward

- Optimized for detecting objects at 0.5-2m range on ground

Use Cases:

- Crate detection (eurobot_perception)

- Pantry detection (eurobot_perception)

- Visual servoing (future)

5.3 Contact Sensors

Left Finger Contact Sensor

Sensor Name: left_finger_contact

Type: contact

Topic: /gripper/left/contact

Monitored Collision: left_finger_collision

Update Rate: 200 Hz

Always On: Yes

Published Message: ros_gz_interfaces/Contacts

contacts: [

{

collision1_name: "simple_robot::left_finger::left_finger_collision"

collision2_name: "crate::collision" (example)

position: Point3D

normal: Vector3D

depth: float

}

]

Contact Detection Logic:

has_contact = len(msg.contacts) > 0

Right Finger Contact Sensor

Sensor Name: right_finger_contact

Specifications: Identical to left sensor

Topic: /gripper/right/contact

Purpose:

- Detect when fingers touch crate

- Enable adaptive grasping (stop closing on contact)

- Confirm successful grip (both fingers in contact)

6. Actuator Systems

6.1 Differential Drive System

Control Method: Velocity-based differential drive

Controlled Joints:

- left_wheel_joint

- right_wheel_joint

Physical Parameters:

Wheel Separation: 0.28 m (280 mm between wheels)

Wheel Radius: 0.04 m (40 mm, 80 mm diameter)

Velocity Limits:

Max Linear Velocity: 1.0 m/s

Max Angular Velocity: 2.0 rad/s (≈115°/s)

Max Linear Acceleration: 2.0 m/s²

Max Angular Acceleration: 3.0 rad/s²

Odometry Publishing:

- Topic: /odom

- Frame: odom → base_link

- Frequency: 50 Hz

Command Interface:

- Topic: /cmd_vel

- Type: geometry_msgs/Twist

linear.x: Forward/backward velocity (m/s)

angular.z: Rotation velocity (rad/s)

Transform Publishing:

- Topic: /tf

- Publishes: odom → base_link transform

- Dead reckoning based on wheel encoders

6.2 Gripper Actuation

Control Method: Position control via ros2_control

Controlled Joints:

- left_finger_joint (prismatic)

- right_finger_joint (prismatic)

Hardware Interface: ign_ros2_control/IgnitionSystem

Controller Type: position_controllers/JointGroupPositionController

Controller Name: gripper_controller

Command Interface:

- Topic: /gripper_controller/commands

- Type: std_msgs/Float64MultiArray

- Format: [left_position, right_position]

- Units: meters

- Range: -0.03 to +0.03 m per finger

State Interfaces:

- Position (current finger positions)

- Velocity (finger motion speed)

- Effort (force applied)

Published States:

- Topic: /joint_states

- Type: sensor_msgs/JointState

- Frequency: 50 Hz

- Includes: left_finger_joint, right_finger_joint positions/velocities

Example Commands:

# Fully open

msg.data = [0.03, 0.03]

# Fully closed

msg.data = [-0.03, -0.03]

# Asymmetric (e.g., during contact-based closing)

msg.data = [0.01, -0.02]  # Left partial, right closed

7. Gazebo Plugins

7.1 Differential Drive Plugin

Plugin: libignition-gazebo-diff-drive-system.so

Name: ignition::gazebo::systems::DiffDrive

Configuration:

<left_joint>left_wheel_joint</left_joint>

<right_joint>right_wheel_joint</right_joint>

<wheel_separation>0.28</wheel_separation>

<wheel_radius>0.04</wheel_radius>

<odom_publish_frequency>50</odom_publish_frequency>

<topic>cmd_vel</topic>

<odom_topic>odom</odom_topic>

<tf_topic>/tf</tf_topic>

<frame_id>odom</frame_id>

<child_frame_id>base_link</child_frame_id>

Functionality:

- Converts /cmd_vel (Twist) to wheel velocities

- Simulates wheel physics and friction

- Publishes odometry (position integration from wheel encoders)

- Publishes TF transforms

7.2 Joint State Publisher Plugin

Plugin: libignition-gazebo-joint-state-publisher-system.so

Name: ignition::gazebo::systems::JointStatePublisher

Published Joints:

- left_wheel_joint

- right_wheel_joint

- left_finger_joint

- right_finger_joint

Topic: /joint_states

Update Rate: 50 Hz

Published Data:

name: [joint names]

position: [joint angles/positions in rad/m]

velocity: [joint velocities in rad/s or m/s]

effort: [joint torques/forces in N⋅m or N]

7.3 Sensors System Plugin

Plugin: libignition-gazebo-sensors-system.so

Name: ignition::gazebo::systems::Sensors

Render Engine: ogre2 (3D graphics rendering)

Functionality:

- Enables all sensor simulations (LiDAR, camera, contacts)

- Manages sensor update loops

- Renders camera images and raycasts LiDAR beams

7.4 Pose Publisher Plugin

Plugin: libignition-gazebo-pose-publisher-system.so

Name: ignition::gazebo::systems::PosePublisher

Configuration:

<publish_link_pose>true</publish_link_pose>

<publish_nested_model_pose>true</publish_nested_model_pose>

<use_pose_vector_msg>true</use_pose_vector_msg>

<static_publisher>true</static_publisher>

<static_update_frequency>10</static_update_frequency>

Functionality:

- Publishes poses of all links

- Useful for debugging and visualization

- Topic: /model/simple_robot/pose (Gazebo-specific)

7.5 Contact System Plugin

Plugin: libignition-gazebo-contact-system.so

Name: ignition::gazebo::systems::Contact

Functionality:

- Enables contact sensor simulation

- Detects collisions on gripper fingers

- Publishes contact messages to specified topics

7.6 ROS 2 Control Bridge Plugin

Plugin: libign_ros2_control-system.so

Name: ign_ros2_control::IgnitionROS2ControlPlugin

Parameters File: /root/eurobot_ws/src/mam_eurobot_2026/models/simple_robot/gripper_controller.yaml

Functionality:

- Bridges Gazebo simulation with ros2_control

- Exposes gripper joints to ros2_control framework

- Enables controller_manager to spawn controllers

8. ROS 2 Control Integration

8.1 Hardware Interface

System Name: GripperSystem

Type: system

Plugin: ign_ros2_control/IgnitionSystem

Controlled Joints:

- left_finger_joint

- right_finger_joint

8.2 Joint Interfaces

Per Joint Configuration

Command Interface:

- Type: position

- Min: -0.03 m

- Max: +0.03 m

State Interfaces:

- position - Current joint position

- velocity - Current joint velocity

- effort - Current joint force/torque

8.3 Controller Manager

Update Rate: 100 Hz

Loaded Controllers:

- joint_state_broadcaster (JointStateBroadcaster)

  - Publishes /joint_states

  - Update rate: 50 Hz

- gripper_controller (JointGroupPositionController)

  - Subscribes: /gripper_controller/commands

  - Controls: Both finger joints simultaneously

  - Update rate: 50 Hz

  - Action monitoring: 20 Hz

9. Coordinate Frames

9.1 Frame Tree

map

└─ odom (published by diff_drive or AMCL)

└─ base_footprint

└─ base_link (canonical, robot center)

├─ left_wheel

├─ right_wheel

├─ caster

├─ lidar_link

├─ camera_link

└─ gripper_base

├─ left_finger

└─ right_finger

9.2 Key Frame Descriptions

| Frame | Purpose | Parent | Transform |
| --- | --- | --- | --- |
| map | Global reference (SLAM/localization) | - | Fixed world frame |
| odom | Odometry reference (drifts over time) | map | From AMCL or odometry |
| base_footprint | Ground-level navigation reference | odom | (0, 0, 0) at ground |
| base_link | Robot center, primary body frame | base_footprint | (0, 0, 0.025) |
| lidar_link | LiDAR sensor frame | base_link | (0, 0, 0.05) |
| camera_link | Camera optical frame | base_link | (0.1, 0, 0.2), pitch=0.7 |
| gripper_base | Gripper mounting frame | base_link | (0.17, 0, -0.02) |
| left_finger | Left gripper finger | gripper_base | (0.04, 0.055+joint, 0) |
| right_finger | Right gripper finger | gripper_base | (0.04, -0.055+joint, 0) |

9.3 Transform Publishers

Static Transforms (fixed joints):

- Published by robot_state_publisher from URDF/SDF

- Includes: base_link → sensors, base_link → gripper_base, etc.

Dynamic Transforms:

- odom → base_link: Published by diff_drive plugin or AMCL

- Gripper joint positions: Updated by ros2_control via joint_states

10. Physical Properties

10.1 Mass Distribution

| Component | Mass (kg) | Percentage |
| --- | --- | --- |
| Base Link | 3.00 | 77.7% |
| Left Wheel | 0.30 | 7.8% |
| Right Wheel | 0.30 | 7.8% |
| Gripper Base | 0.30 | 7.8% |
| Left Finger | 0.08 | 2.1% |
| Right Finger | 0.08 | 2.1% |
| LiDAR | 0.10 | 2.6% |
| Camera | 0.05 | 1.3% |
| Caster | 0.05 | 1.3% |
| Total | 3.86 kg | 100% |

10.2 Inertia Tensors

Base Link (largest contributor):

Ixx = 0.0186 kg⋅m² (roll inertia)

Iyy = 0.0228 kg⋅m² (pitch inertia)

Izz = 0.0391 kg⋅m² (yaw inertia)

- Higher Izz reflects wider base (harder to rotate in yaw)

Wheels (rotating masses):

Ixx = Iyy = 0.00024 kg⋅m² (bending resistance)

Izz = 0.00048 kg⋅m² (spin inertia, 2× radial)

10.3 Friction Coefficients

| Surface | μ (static/kinetic) | Purpose |
| --- | --- | --- |
| Wheels | 1.0 | High traction, no slip |
| Caster | 0.01 | Free rolling, minimal drag |
| Gripper Fingers | 8.0 | Secure grip, prevent object slip |

10.4 Contact Parameters

Wheels and Caster:

- Stiffness (kp): 1×10⁶ N/m (rigid contact)

- Damping (kd): 100 N⋅s/m (prevent bouncing)

Gripper Fingers:

- Stiffness: 1×10⁶ N/m

- Max velocity: 0.01 m/s (slow penetration)

- Min depth: 0.001 m (1mm sensitivity)