Eurobot Perception Package Documentation

1. Package Overview

High-Level Goal

The eurobot_perception package provides vision-based object detection capabilities for an autonomous robot competing in the Eurobot competition. The system detects colored crates (blue and yellow) and green pantries in real-time using camera input, converting pixel-space detections into robot-relative coordinates for navigation and manipulation planning.

System Overview

The perception system uses computer vision techniques to identify and localize game objects in the robot's environment. It processes camera images through HSV color filtering, contour detection, and geometric transformation to produce actionable detection data that downstream systems can use for task planning and navigation.

Key capabilities:

- Real-time detection of blue and yellow crates

- Detection of green pantries for crate placement

- Distance and angle estimation using pinhole camera model

- Robot-relative coordinate conversion (meters)

- Configurable detection parameters

- Debug visualization support

Hardware and Testing Environment

- Simulation Platform: Gazebo

- ROS Version: ROS 2 Humble

- Sensors: RGB Camera (simulated)

- Visualization: RViz,

2. Package Summary

Package Name

eurobot_perception

Goal

Provide robust, real-time perception capabilities for detecting game objects (crates and pantries) and converting visual information into spatial data usable by navigation and task management systems.

Dependencies

- rclpy - ROS 2 Python client library

- sensor_msgs - Standard ROS image messages

- std_msgs - Standard message headers

- cv_bridge - Converts between ROS and OpenCV images

- eurobot_interfaces - Custom message definitions for detections

- OpenCV (cv2) - Computer vision processing

- NumPy - Numerical operations

Main Nodes

| Node Name | Purpose |
| --- | --- |
| crate_perception | Detects blue and yellow crates, estimates distance and position |
| pantry_detection | Detects green pantries for crate placement targets |

Configuration Files

No dedicated YAML configuration files are currently used. All parameters are declared programmatically within the nodes and can be overridden via command-line or launch file parameters.

Launch Files

No launch files are currently implemented. Nodes are executed individually using ros2 run commands.

Interaction with Other Packages

The perception package publishes detection data that would  be consumed by:

- Task Manager - Receives crate and pantry locations to plan pickup/placement sequences

- Navigation - Uses object positions to plan paths and avoid obstacles

- Manipulator - Uses precise object coordinates for gripper positioning

3. Node-Level Explanation

3.1 Crate Perception Node

Node Name

crate_perception

Goal / Functionality

Detects blue and yellow crates in the camera field of view, estimates their distance and bearing relative to the robot, and publishes structured detection messages containing robot-relative coordinates in meters.

Subscribed Topics

| Topic | Message Type | Purpose |
| --- | --- | --- |
| /camera/image_raw | sensor_msgs/Image | Raw camera feed for processing (configurable) |

Published Topics

| Topic | Message Type | Purpose |
| --- | --- | --- |
| /crate/detections | eurobot_interfaces/CrateDetectionArray | Array of detected crates with position, color, and confidence |
| /crate/image_debug | sensor_msgs/Image | Annotated debug visualization (only if visualize=True) |

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| camera_topic | string | /camera/image_raw | Input camera topic name |
| visualize | bool | True | Enable debug image publishing |
| min_area | int | 2000 | Minimum contour area (pixels) to consider valid |
| known_crate_width | float | 0.12 | Physical width of crate in meters |
| focal_length_px | float | 600.0 | Camera focal length in pixels (calibration) |
| camera_center_offset | float | 0.0 | Camera mounting offset from robot center (meters) |

Main Logic Flow

- Image Acquisition: Receives RGB image from camera topic and converts to OpenCV format using cv_bridge

- Color Space Conversion: Converts BGR to HSV color space for robust color detection

- Color Filtering: For each target color (blue, yellow):

  - Creates binary mask using HSV range thresholds

  - Applies morphological closing to reduce noise

- Contour Detection: Finds external contours in each color mask

- Filtering: Rejects contours smaller than min_area threshold

- Geometric Analysis: For each valid contour:

  - Computes bounding rectangle and centroid

  - Estimates distance using pinhole camera model: distance = (object_width × focal_length) / pixel_width

  - Calculates bearing angle from image center using arctan2

  - Converts to robot-relative Cartesian coordinates (x forward, y lateral)

  - Computes confidence score based on relative contour area

- Message Publishing: Packages all detections into CrateDetectionArray and publishes

- Visualization: If enabled, annotates frame with bounding boxes and coordinate labels

Color Detection Ranges (HSV)

Blue:   H: [95-135],  S: [80-255],  V: [40-255]

Yellow: H: [20-35],   S: [100-255], V: [100-255]

Coordinate Transformation

The node converts from pixel coordinates to robot-relative meters:

distance = (crate_width × focal_length) / pixel_width

angle_rad = atan2(pixel_offset_x, focal_length)

robot_x = distance × cos(angle_rad) + camera_offset

robot_y = distance × sin(angle_rad)

Where robot_x is forward and robot_y is lateral offset.

3.2 Pantry Detection Node

Node Name

pantry_detection_node

Goal / Functionality

Detects green-colored pantries (target zones for crate placement) in the camera view, assigns unique IDs to each pantry, and publishes their robot-relative positions. Implements duplicate detection filtering to avoid re-detecting the same pantry.

Subscribed Topics

| Topic | Message Type | Purpose |
| --- | --- | --- |
| /camera/color/image_raw | sensor_msgs/Image | RGB camera feed (configurable) |

Published Topics

| Topic | Message Type | Purpose |
| --- | --- | --- |
| /pantry/detections | eurobot_interfaces/PantryDetectionArray | Array of detected pantries with ID and position |

Parameters

| Parameter | Type | Default | Description |
| --- | --- | --- | --- |
| camera_topic | string | /camera/color/image_raw | Input camera topic |
| debug_view | bool | True | Display OpenCV debug window |
| green_lower_hsv | int[3] | [40, 40, 40] | Lower HSV threshold for green |
| green_upper_hsv | int[3] | [80, 255, 255] | Upper HSV threshold for green |
| min_area | int | 5000 | Minimum contour area in pixels |
| known_pantry_width | float | 0.15 | Physical width of pantry in meters |
| focal_length_px | float | 600.0 | Camera focal length in pixels |
| camera_center_offset | float | 0.0 | Camera offset from robot center |

Main Logic Flow

- Image Reception: Receives and converts RGB image to OpenCV format

- HSV Conversion: Converts to HSV color space

- Green Masking: Creates binary mask for green color range

- Morphological Operations:

  - Opening to remove small noise

  - Closing to fill gaps

- Contour Extraction: Finds external contours in mask

- Size Filtering: Rejects contours below min_area

- Position Calculation: For each valid contour:

  - Computes centroid and bounding box

  - Estimates distance using pinhole model

  - Calculates bearing angle

  - Transforms to robot-relative coordinates (x, y in meters)

- Duplicate Filtering: Checks if pantry location is within 0.5m of previously detected pantries

- ID Assignment: Assigns incremental unique ID to new pantries

- Publishing: Publishes PantryDetectionArray with all new detections

- Visualization: If debug enabled, displays annotated frame with OpenCV window

Duplicate Detection Logic

# Avoid re-detecting the same pantry

if not any(hypot(rel_x - px, rel_y - py) < 0.5

for px, py in detected_pantries):

# This is a new pantry

assign_new_id()

4. Topic Communication Summary

System-Wide Topic Flow

| Node | Publishes | Subscribes | Message Type | Description |
| --- | --- | --- | --- | --- |
| crate_perception | /crate/detections | /camera/image_raw | CrateDetectionArray | Crate positions with color, distance, angle |
| crate_perception | /crate/image_debug | - | sensor_msgs/Image | Debug visualization (optional) |
| pantry_detection_node | /pantry/detections | /camera/color/image_raw | PantryDetectionArray | Pantry positions with unique IDs |

Message Definitions

CrateDetection.msg

string color              # "blue" or "yellow"

float32 x                 # x coordinate in robot frame (meters, forward)

float32 y                 # y coordinate in robot frame (meters, lateral)

float32 distance          # estimated distance to crate in meters

float32 angle             # angle relative to robot heading in degrees

float32 confidence        # detection confidence (0.0 - 1.0)

CrateDetectionArray.msg

std_msgs/Header header

CrateDetection[] detections

PantryDetection.msg

int32 id                  # unique pantry ID

float32 x                 # x coordinate in robot frame (meters, forward)

float32 y                 # y coordinate in robot frame (meters, lateral)

float32 distance          # estimated distance to pantry in meters

float32 angle             # angle relative to robot heading in degrees

float32 confidence        # optional: detection confidence (0.0 - 1.0)

PantryDetectionArray.msg

std_msgs/Header header

PantryDetection[] detections

Detection Event Flow

- Continuous Monitoring: Both perception nodes continuously process incoming camera frames

- Independent Detection: Crate and pantry detection operate independently on separate topics

- Real-Time Publishing: Detections are published immediately when objects are found

- Downstream Consumption: Detection messages are designed to be consumed by higher-level planning systems

- Coordinate System: All positions are in robot-relative frame (x forward, y lateral, in meters)

Expected Runtime Behavior

Crate Detection:

- Node starts and subscribes to camera topic

- When blue or yellow objects appear in view:

  - Publishes CrateDetectionArray messages

  - Debug image shows green bounding boxes with labels like "blue 1.23m (1.20,0.15)"

- No output if no crates are visible

Pantry Detection:

- Node starts and subscribes to camera topic

- When green objects appear:

  - Assigns unique incremental IDs (Pantry 1, Pantry 2, ...)

  - Publishes PantryDetectionArray messages

  - OpenCV window shows annotated detections

  - Filters duplicate detections within 0.5m radius

5. Simplified System Explanation

The eurobot_perception package gives the robot "eyes" to see game objects. It uses a camera to capture images, then applies color filtering to find blue and yellow crates or green pantries. Once an object is spotted, the system calculates how far away it is and at what angle, converting pixel positions into real-world distances in meters. This information is packaged into messages that tell the robot "there's a blue crate 1.2 meters ahead and 0.3 meters to the left" or "there's a pantry 2 meters in front." These messages can then be used by other parts of the robot to decide what to do next like driving toward a crate to pick it up or navigating to a pantry to place it.

The system is designed to work in real-time, processing every camera frame to keep track of objects as the robot moves around. It's smart enough to filter out noise (like small blobs that aren't real objects) and, for pantries, remembers which ones it's already seen to avoid counting the same pantry twice. All of this runs independently for crates and pantries, making it flexible and modular.

8. Technical Details

Computer Vision Techniques

Color Space Selection: HSV (Hue, Saturation, Value) is used instead of RGB because it's more robust to lighting variations. The hue channel isolates color independent of brightness.

Morphological Operations:

- Closing: Fills small holes in detected regions (kernel 5×5, 2 iterations for crates)

- Opening + Closing: Removes noise then fills gaps (pantry detection)

Distance Estimation: Uses the pinhole camera model:

distance = (real_object_width × focal_length_pixels) / object_width_pixels

This assumes the camera is calibrated and the object width is known.

Coordinate Transformation: Converts from camera-centric pixel coordinates to robot-relative Cartesian coordinates:

angle_rad = atan2(pixel_offset_from_center, focal_length)

robot_x = distance × cos(angle_rad) + camera_offset

robot_y = distance × sin(angle_rad)

Limitations

- Occlusion: Cannot detect objects blocked by others

- Lighting Sensitivity: HSV ranges may need adjustment for different environments

- Single Camera: No depth information, relies on geometric assumptions

- Color Uniqueness: Assumes target colors don't appear elsewhere in scene

- Scale Ambiguity: Distance estimation requires known object size

Appendix A: Package Structure

eurobot_perception/

├── package.xml

├── setup.py

├── resource/

│   └── eurobot_perception

├── eurobot_perception/

│   ├── __init__.py

│   ├── crate_perception.py

│   └── pantry_detection.py

└── test/

├── test_copyright.py

├── test_flake8.py

└── test_pep257.py

Appendix B: Dependencies Installation

# ROS 2 dependencies (should be installed with ros-humble-desktop)

sudo apt install ros-humble-sensor-msgs ros-humble-cv-bridge

# Python dependencies

pip3 install opencv-python numpy

# Custom interfaces (build from source)

cd ~/eurobot_ws/src

# Ensure eurobot_interfaces package exists

colcon build --packages-select eurobot_interfaces